Chapter

.c.	General System Services

.i.system information;

.c.:System information

.i.system information:o/s version;

.i.version;

.i.p_version;	.c.::Get the operating system version (p_version)

.f.UINT p_version(VOID);

Return the operating system version number.

The returned version number should be interpreted as a 4─digit hexadecimal number of the form:

X.YYZ

where X is the major release number, YY is the minor release number and Z is normally the hexadecimal digit F (internal releases use A and B to designate alpha and beta releases, respectively).

For example, a return of 0x123F is interpreted as 1.23F.

.i.system information:ROM version;

.i.ROM:version;

.i.p_romversion;	.c.::Get the ROM version (p_romversion)

.f.UINT p_romversion(VOID);

Return the ROM version number.

The ROM contains the operating system and other system components such as, for example, the window server. The tool used to build the ROM requires a version number to be specified and this is the version number that is retrieved by this function.

See p_version above for the interpretation of the version number.

.i.system information:last shutdown,cause;

.i.p_getres;	.c.::Get the cause of the last system shut-down (p_getres)

.f.INT p_getres(VOID);

Return a number indicating the cause of the last system shut-down, as follows:

E_IS_A_COLD_START

The system started up for the first time (or after a period during which all power had been removed, including the Lithium back-up).

E_IS_A_POWERFAIL_START

.i.hardware interrupts;

The hardware forced a shut-down because the voltage got too low. This should not happen because the system software gets a non-maskable interrupt if the voltage drops below a certain threshold (but not low enough for the hardware to force a shut-down) and this interrupt code automatically switches the hardware off. However, if the clean─up takes too long because of a poorly designed device driver, the hardware will force the machine off before the interrupt completes - in which case p_getres returns E_IS_A_POWERFAIL_START. The environment variables and the contents of M: are preserved.

E_IS_A_RESET_START

The user pressed the recessed reset button. The environment variables and the contents of M: will have been preserved unless the ESC key was held down during the reset in which case they are both cleared.

E_IS_A_KERNEL_FAULT

The system was reset because a serious fault occurred while executing in the operating system kernel. This could result from: (1) a bug in the operating system or a system process; (2) a program bug that managed to overcome the operating system's defences or (3) a hardware problem such as a RAM fault. The environment variables and the contents of M: will be preserved (unless the system detects a memory corruption).

E_IS_A_NEW_OS_START

The system was reset after programming a new operating system into the Flash ROM (normally after running the repro program). The environment variables and the contents of M: have been cleared.

.i.system information:o/s data;

.i.p_getosd;	.c.::Get operating system data (p_getosd)

.f.VOID p_getosd(VOID *pTarget, VOID *pSource, UINT length);

Copy length bytes from offset pSource in the operating system data space to pTarget.

Operating system handles (such as memory segment handles and semaphore handles) are actually addresses in the operating system data space of the appropriate control entry.

If you AND a process ID with E_PID_MASK you get the address in operating system space of the corresponding process control entry - as described in the chapter Processes and Inter-Process Messaging.

.i.system information:power supply type;

.i.power supply;

.i.p_getpsu;	.c.::Get power supply type (p_getpsu)

.f.INT p_getpsu(VOID);

.i.header files:epoc.h;

.i.epoc.h;

Return one of the following values, defined in epoc.h, to indicate the power supply type on a SIBO machine:

E_PSU_OLD

E_PSU_MAXIM

E_PSU_S3

E_PSU_S3_A9

Apart from this service EPOC hides the differences between the various power supplies.

Machines in the MC GI range of SIBO computers, for example, may use either the E_PSU_OLD or the E_PSU_MAXIM power supply variants, each of which requires a slightly different variant of the EPOC operating system. The REPRO program that blows a new operating system into the Flash ROM of the MC GI range uses this service to blow the appropriate variant of EPOC.

.i.system information:language code;

.i.system information:country code;

.i.country code;

.i.language code;

.c.:Language and country

SIBO machines are produced in a number of language variants, differing in the following respects:

■	the language code

■	the language of the text used by the ROM-based software (for example, the error messages returned by p_errs and the month names returned by p_nmmon)

■	character type and conversion tables as described in the chapter Characters, Strings, Buffers and Queues

■	the keyboard layout

■	the default country and country-dependent data

A particular language variant will always have a different language code and text but may not differ in all the above.

.i."ROM::":SYS$CTRY.CFO;

.i.SYS$CTRY.CFO;

The system was designed to be produced in a variety of languages and, as far as the system services are concerned, all the above variations are encapsulated in a single configuration file in the ROM called ROM::SYS$CTRY.CFO. Depending on the machine, there may be further files (for example, "resource files") containing language-dependent data for higher level system components.

Unlike the language-dependent data, the country-dependent data may be altered from the language-dependent defaults.

.i.system information:language code;

.i.p_getlanguage;	.c.::Get the language code (p_getlanguage)

.f.INT p_getlanguage(VOID);

.i.ROM:configuration file;

Return the language code from the ROM configuration file.

.i.header files:config.h;

.i.config.h;

The language code can be used by applications that contain the text for more than one language to determine which language to present. The language codes, defined in p_config.h, are as follows:

1 = English

2 = French

3 = German

4 = Spanish

5 = Italian

6 = Swedish

7 = Danish

8 = Norwegian

9 = Finnish

10 = USA

11 = Swiss french

12 = Swiss German

13 = Portuguese

14 = Turkish

15 = Icelandic

16 = Russian

17 = Hungarian

18 = Dutch

19 = Belgian Flemish

20 = Australian

21 = New Zealand

22 = Austrian

23 = Belgian french

.i.system information:o/s text;

.i.p_gettext;	.c.::Get operating system text (p_gettext)

.f.INT p_gettext(INT n, TEXT *pBuffer);

Get the nth string from the ROM configuration file and write it to pBuffer.

Returns zero if successful or the negative E_GEN_ARG if n is outside the range of the text strings in the configuration file.

This function is called by specific text retrieval functions such as p_errs and p_nmmon.

Applications only need to use p_gettext when retrieving text associated with a higher level of system software (in which case the documentation of the higher level software will list appropriate values of n).

.i.system information:country-dependent data,fetch;

.i.p_getctd;	.c.::Get country-dependent data (p_getctd)

.f.VOID p_getctd(.i.structures:E_CONFIG; *pcfg);

.i.header files:config.h;

.i.config.h;

Write a copy of the system E_CONFIG struct to pcfg where the E_CONFIG struct is defined in p_config.h as:

typedef struct

{

UWORD countryCode;

WORD gmtOffset;

UBYTE dateType;

UBYTE timeType;

UBYTE currencySymbolPosition;

UBYTE currencySpaceRequired;

UBYTE currencyDecimalPlaces;

UBYTE currencyNegativeInBrackets;

UBYTE currencyTriadsAllowed;

UBYTE thousandsSeparator;

UBYTE decimalSeparator;

UBYTE dateSeparator;

UBYTE timeSeparator;

UBYTE currencySymbol[9];

UBYTE startOfWeek;

UBYTE summerTime;

UBYTE clockType;

UBYTE dayAbbreviation;

UBYTE monthAbbreviation;

UBYTE workDays;

UBYTE units;

UBYTE spare[9];

} .i.structures:E_CONFIG;

The countryCode specifies a country by its international dialling code.

See the description of p_getctd in the chapter Time, Timers and Dates for a description of the time-related fields: gmtOffset, dateType, timeType, dateSeparator, timeSeparator, startOfWeek, summerTime, clockType, dayAbbreviation, monthAbbreviation and workDays.

See the description of p_getctd in the chapter Floating Point for a description of the fields that are related to the display of floating point numbers and currency: currencySymbol, currencySymbolPosition, currencySpaceRequired, currencyDecimalPlaces, currencyNegativeInBrackets, currencyTriadsAllowed, thousandsSeparator and decimalSeparator.

.i.system:country-dependent data,set;

.i.p_setctd;	.c.::Set country-dependent data (p_setctd)

.f.VOID p_setctd(.i.structures:E_CONFIG; *pcfg);

Sets the country-dependent data from the E_CONFIG structure pointed to by pcfg.

When changing a particular field or fields you would normally:

■	use p_getctd to get a copy of the E_CONFIG struct

■	modify the field or fields, as required

■	use p_setctd to write back the modified E_CONFIG struct

.i.system information:switch on;

.i.system information:switch off;

.i.switch on;

.i.switch off;

.c.:Switching on and off

.i.auto switch off;

By default, the auto-switch-off period is set to 300 seconds. The auto-switch-off period may be sensed and set by calling p_getauto and p_setauto respectively. Auto-switch-off when a mains adaptor is connected may be disabled by calling p_setautomains, and the corresponding state sensed by p_getautomains.

.i.process:null;

.i.process:zero priority;

.i.null process;

.i.zero priority process;

The zero priority null process automatically switches the machine off to reduce power consumption after the system has been inactive for the auto-switch-off period. In situations where the null process does not get an opportunity to run, a process can assume responsibility for allowing the machine to switch off by calling p_allowoff.

Some processes are marked as not being significant when it comes to determining what constitutes activity. For example, a continuously running clock program should not stop the system from automatically switching off in the absence of any significant activity. See p_unmarka and p_marka in the chapter Processes and Inter-Process Messaging for additional details on how to avoid keeping the machine switched on by continuously running applications.

.i.system:switch off;

.i.p_off;	.c.::Switch off (p_off)

.f.VOID p_off(UINT uTime);

Switch off the machine indefinitely or for any time up to approximately 4.5 hours and return when the machine switches on.

If uTime is 0xffff the machine switches off indefinitely and will stay off until an outstanding absolute timer expires or the user switches on the machine. Equivalent to the machine automatically switching off or being switched off by the user.

If uTime is greater than 8, the machine switches off for up to uTime 1/4ths of a second (although it will still switch on if an outstanding absolute timer expires or the user switches on the machine). (If uTime is less than or equal to 8, calling p_off has no effect.)

On the IBM PC version of EPOC, calling p_off has no effect.

.i.system information:auto-switch-off period,fetch;

.i.p_getauto;	.c.::Get the auto-switch-off period (p_getauto)

.f.INT p_getauto(VOID);

Return the current auto-switch-off period in seconds.

If the auto-switch-off period is 0xffff, the system does not automatically switch off.

.i.system:auto-switch-off period,set;

.i.p_setauto;	.c.::Set the auto-switch-off period (p_setauto)

.f.VOID p_setauto(INT n);

Set the auto-switch-off period to n seconds.

Passing an n of -1 (0xffff) stops the system from automatically switching off.

Calling p_setauto with n less than 15 is equivalent to calling p_setauto(15).

.i.system information:switch-off if mains,fetch;

.i.p_getautomains;	.c.::Get switch-off state when mains is present (p_getautomains)

.f.INT p_getautomains(VOID);

This function is only available in EPOC version 3.18 or later.

Return TRUE if auto-switch-off is disabled when mains is present, otherwise return FALSE.

.i.system:switch-off if mains,enable;

.i.system:switch-off if mains,disable;

.i.p_setautomains;	.c.::Disable/enable switch-off if mains is present (p_setautomains)

.f.VOID p_setautomains(INT flag)

This function is only available in EPOC version 3.18 or later.

Disable or enable auto-switch-off if mains is present.

If flag is TRUE, auto-switch-off is disabled while mains is present and if flag is FALSE, auto-switch-off is enabled.

Even if enabled, the machine will not switch off when mains is absent if switch-off has been stopped by use of p_setauto.

.i.system:auto-switch-off,allow;

.i.p_allowoff;	.c.::Allow auto switch off (p_allowoff)

.f.VOID p_allowoff(VOID);

Allow the machine to switch off if the auto-switch-off period has expired.

A compute-bound process which has marked itself as non-active (by calling p_unmarka) does not allow the machine to switch off since the null process will never get an opportunity to run. Such a process should call p_allowoff from time to time. There is no particular advantage in calling p_allowoff more frequently than at intervals of 15 seconds - the shortest auto-switch-off period.

.i.EPOC:PC;

On the IBM PC version of EPOC, calling p_allowoff has no effect.

.i.system:ON key event,enable;

.i.system:ON key event,disable;

i.p_setonevent;	.c.::Enable/disable the ON key event (p_setonevent)

.f.VOID p_setonevent(INT state)

This function is only available in EPOC version 2.28 or later.

.i.event;

Enables or disables the event that is sent to the window server when the ON key is pressed.

If state is FALSE, the event is disabled, any other value enables the event.

.i.system information:power supply;

.i.power supply;

.i.battery:type;

.i.battery:voltage level;

.i.battery:backup;

.i.mains adaptor;

.c.:Power supply

This section describes functions to:

■	determine the presence or absence of the main battery, the Lithium backup battery or the mains adaptor (p_supplyinfo)

■	get the voltage level of the main battery (or mains adaptor, if present) and the Lithium backup battery (p_supply)

■	determine whether the mains adaptor is connected (p_supply)

■	get the nominal maximum voltages of the main battery and the Lithium backup battery (p_wsupply)

■	get the recommended low voltage warning levels for the main battery and the Lithium backup battery (p_wsupply)

■	get the time and date of insertion of the main battery, and information about main battery usage (p_supplyinfo)

■	sense and set the main battery type (p_getbat and p_setbat respectively)

The main battery type is only significant on machines that can take more than one battery type (such as the MC GI range) and affects the voltages returned by p_wsupply.

The main battery types are as follows:

.i.E_BATTERY_UNKNOWN

the battery type is initially set to this value when EPOC starts up (equivalent in its effect to E_BATTERY_ALKALINE)

.i.E_BATTERY_ALKALINE

the battery is an Alkaline

.i.E_BATTERY_NICAD_600

the battery is a 600 mA hour NiCd rechargeable

.i.E_BATTERY_NICAD_1000

the battery is a 1000 mA hour NiCd rechargeable

Since the hardware is unable to identify automatically the battery type it is the responsibility of the user to set the battery type to match that actually fitted. The E_BATTERY_UNKNOWN value is intended to trigger the system into prompting the user to identify the battery type.

.i.system information:power status,fetch;

.i.p_supply;	.c.::Get power supply status (p_supply)

.f.VOID p_supply(.i.structures:E_SUPPLY; *pValue);

Write the status of the various supplies to the E_SUPPLY struct at pValue where E_SUPPLY is defined as:

typedef struct

{

UWORD mainBatteryReading;

UWORD lithiumBatteryReading;

WORD mainsPresent;

} .i.structures:E_SUPPLY;

where:

mainBatteryReading

is the main battery voltage in millivolts (or the mains adaptor voltage if the mains adaptor is present)

lithiumBatteryReading

is the Lithium backup battery voltage in millivolts

mainsPresent

is negative if the mains adaptor status cannot be determined (because the SSD pack doors are open); zero if the mains adaptor is not present; one if the mains adaptor is present

.i.system information:power status additional,fetch;

.i.p_supplyinfo;	.c.::Get additional power supply data (p_supplyinfo)

.f.VOID p_supplyinfo(.i.structures:E_SUPPLY_INFO; *pValue);

This function is only available in EPOC version 3.18 or later.

Write information concerning the various power supplies to the E_SUPPLY_INFO struct at pValue.

.i.header files:epoc.h;

.i.epoc.h;

The E_SUPPLY_INFO is defined in epoc.h as:

typedef struct

{

UBYTE mainBatteryLevel;

UBYTE mainBatteryStatus;

UBYTE backupBatteryLevel;

UBYTE dcLevel;

UWORD warningFlags;

ULONG insertionDate;

ULONG ticksInUseBattery;

ULONG ticksInUseDc;

ULONG maTicks;

} .i.structures:E_SUPPLY_INFO;

where:

mainBatteryLevel

is TRUE if the main battery is detected to be present

mainBatteryStatus

is the main battery status and can be one of: E_MBAT_ZERO, E_MBAT_VERY_LOW, E_MBAT_LOW or E_MBAT_GOOD

backupBatteryLevel

is TRUE if the Lithium backup battery is detected to be present

dcLevel

is TRUE if the mains adaptor is detected to be present

warningFlags

can be one of: E_SUPPLY_SOUND_WARNING, E_SUPPLY_FLASH_WARNING or E_SUPPLY_SYSTEM_TIME_CHANGED

insertionDate

is the system time at which the current main battery was inserted

ticksInUseBattery

is the total length of time in 'ticks' (1/32 second) for which the main battery has been present, not including any time when a mains adaptor has been present

ticksInUseDc

is the length of time in 'ticks' (1/32 second) for which the main battery has been present with its voltage level below level 1. This does not include any time when a mains adaptor has been present

maTicks

is the length of time in 'ticks' (1/32 second) for which the main battery has been present with its voltage level below level 2. This does not include any time when a mains adaptor has been present

.i.system information:battery warning;

.i.system information:maximum levels;

.i.p_wsupply;	.c.::Get battery warning and maximum levels (p_wsupply)

.f.VOID p_wsupply(.i.structures:E_SUPPLY_WARNINGS; *pValue);

Write the recommended low voltage warning level and the nominal maximum voltage of the main battery and the Lithium backup battery to pValue.

The E_SUPPLY_WARNINGS struct is defined as:

typedef struct

{

UWORD mainBatteryWarning;

UWORD lithiumBatteryWarning;

UWORD mainBatteryMax;

UWORD lithiumBatteryMax;

} .i.structures:E_SUPPLY_WARNINGS;

where:

mainBatteryWarning

is the recommended voltage at which to warn the user of a low main battery

lithiumBatteryWarning

is the recommended voltage at which to warn the user of a low Lithium backup battery

mainBatteryMax

is the nominal maximum voltage of the main battery

lithiumBatteryMax

is the nominal maximum voltage of the Lithium backup battery

All voltages are provided in millivolts.

If the host machine can take more than one battery type, the voltages will depend on the battery type set with p_setbat.

.i.system information:battery type,get;

.i.p_getbat;	.c.::Get the battery type (p_getbat)

.f.INT p_getbat(VOID);

Return the current battery type.

The battery types of the form E_BATTERY_xxx are described at the beginning of this section.

.i.system:battery type,set;

.i.p_setbat;	.c.::Set the battery type (p_setbat)

.f.INT p_setbat(INT nType);

Set the battery type to nType and return zero if successful or E_GEN_NSUP if battery type nType is not supported by the hardware.

The battery types of the form E_BATTERY_xxx are described at the beginning of this section.

.i.system information:keyboard;

.i.keyboard;

.c.:Keyboard

.i.system information:state of keys,fetch;

.i.p_getscancodes;	.c.::Get the state of all keys (p_getscancodes)

.f.INT p_getscancodes(UWORD *pScan);

This function is only available in EPOC version 3.18 or later.

Write the current state of all the keys on the keyboard to the array of ten words pointed to by pScan.

.i.SIBO;

Each key corresponds to a bit in the word array. In the case of the Series 3a, the lowest eleven bits in each of the first eight words are used. All other machines in the SIBO range use the lowest eight bits in all ten words. If a key is depressed the corresponding bit will be set, otherwise it is clear.

The mapping between keys and bits in the array varies from machine to machine. In all cases, however, if no key is depressed all ten words in the array will contain zero. The following example, to detect if any key is pressed, is suitable for use on all SIBO machines:

GLDEF_C IsKeyDown(VOID)

{

UWORD *p;

UWORD scans[10];

p=&scans[0];

p_getscancodes(p);

for (;p<=&scans[9];p++)

{

if (*p)

return(TRUE);

}

return(FALSE);

}

The array index and bit index for the keys on the Series 3a are recorded in the file epockeys.inc.

.c.:Display

.i.system information:display type,fetch;

.i.p_getlcd;	.c.::Get the system display type (p_getlcd)

.f.INT p_getlcd(VOID);

.i.header files:epoc.h;

.i.epoc.h;

Return the (positive) system display type, where the display types are defined in epoc.h.

The function returns -1 if the host is a PC that has an unknown display type.

On a SIBO machine the display type is a good way of .i.identify machine type;

.i.machine type;

identifying the model. Examples are:

E_LCD_640_400

E_LCD_640_200

E_LCD_160_80

E_LCD_240_80

E_LCD_480_160

a 640 by 400 pixel display as on the MC 400

a 640 by 200 pixel display as on the MC 200

a 160 by 80 pixel display as on the HC

a 240 by 80 pixel display as on the Series 3

a 480 by 160 pixel display as on the Series 3a

On a PC, the display types are:

E_PC_HERC

E_PC_CGA

E_PC_MDA

E_PC_EGA_MONO

E_PC_EGA_COLOUR

E_PC_VGA_MONO

E_PC_VGA_COLOUR

Hercules graphics adaptor

CGA graphics adaptor

MDA display adaptor

EGA monochrome graphics adaptor

EGA colour graphics adaptor

VGA monochrome graphics adaptor

VGA colour graphics adaptor

If p_getlcd returns -1 (which can only happen on a PC), and you choose not to fail, it is recommended that you proceed as if it had returned E_PC_VGA_MONO.

.i.system:LCD contrast,change;

.i.p_lcdcontrastdelta;	.c.::Change the LCD contrast (p_lcdcontrastdelta)

.f.VOID p_lcdcontrastdelta(INT nDelta);

Step the LCD contrast up or down depending on whether nDelta is positive or negative respectively (the magnitude of nDelta is ignored.

The LCD contrast is changed through a sequence of values in a circular fashion. That is, stepping the LCD contrast up when it is already at its maximum value sets it to its minimum value and vice versa.

The sequence of values that may be set varies from model to model. On a particular model, the values can be ascertained by using p_getlcdcontrast, described below.

.i.system information:LCD contrast,get;

.i.p_getlcdcontrast;	.c.::Get the current LCD contrast (p_getlcdcontrast)

.f.INT p_getlcdcontrast(VOID)

Return the current LCD contrast setting.

.i.system:backlight on;

.i.system:backlight off;

.i.p_backlight;	.c.::Switch the backlight on or off (p_backlight)

.f.INT p_backlight(INT mode);

Switch the backlight on or off, depending on mode as follows:

E_BACKLIGHT_OFF

switch the backlight off (if it is not already off)

E_BACKLIGHT_ON

switch the backlight on (if it is not already on) and reset the auto-switch-off timer

E_BACKLIGHT_TOGGLE

switch the backlight on and reset the auto-switch-off timer if it is currently off or switch the backlight off if it is currently on

E_BACKLIGHT_QUERY

does nothing and is used just to get the current backlight on/off state

The function always returns the on/off state (TRUE if on, FALSE if off) as it was as the function was entered.

.i.system:backlight control,set;

.i.p_setbacklight;	.c.::Set the backlight control value (p_setbacklight)

.f.VOID p_setbacklight(UINT flag);

Enable or disable the backlight key and set the backlight auto-switch-off interval in ticks.

If the most significant bit of flag is set (as given by the bit mask E_BACKLIGHT_DISABLE), the operating system will not respond to the backlight key. (However, this does not stop the backlight from being switched by a program calling p_backlight.)

The lower 15 bits of flag specifies the backlight auto-switch-off interval in ticks (1/32ths of a second). If this value is zero, the backlight is not automatically switched off by a backlight timer and it will remain on until the machine switches off.

For example:

p_setbacklight(96);

sets the backlight auto-switch-off interval to 3 seconds and:

p_setbacklight(E_BACKLIGHT_DISABLE|(32*5));

sets the backlight auto-switch-off interval to 5 seconds and disables the backlight key.

.i.system information:backlight enablement,fetch;

.i.p_getbacklight;	.c.::Get the backlight enablement (p_getbacklight)

.f.UINT p_getbacklight(VOID);

Return the backlight control value as set by p_setbacklight, described above.

.i.system information:sound;

.i.sound;

.c.:Sound

.i.piezo-electric;

Most SIBO machines contain a piezo-electric buzzer in addition to a loudspeaker.

This section describes how to use the piezo to make a sound, and how to manipulate the set of flags used to control the sound produced by the system.

.i.device driver:"SND:";

.i."SND:";

The piezo uses very little power and is an easy way of generating sound, although it is fairly quiet. Consider using the  SND: device driver, which drives the loudspeaker, if greater sound complexity or a louder sound is required. The SND: device driver is described in the I/O Devices Reference manual.

The bit masks for the flags controlling sound output are:

E_SOUND_KEYBOARD

keyboard clicks are silenced if clear

E_SOUND_BUZZER

the piezo sound system is silenced (except for keyclicks) if clear

E_SOUND_DEVICE

the SND: device driver is silenced if clear

E_SOUND_LOUD

the piezo will sound louder if set

E_SOUND_DISABLE

all sound in the system is silenced if set

.i.piezo-electric:sound,make;

.i.sound:piezo-electric;

.i.system:sound,piezo-electric;

.i.p_sound;	.c.::Make a sound with the piezo (p_sound)

.f.VOID p_sound(UINT nDuration, UINT nPitch);

Make a sound through the piezo for nDuration system ticks and at pitch nPitch (where the pitch has frequency 512/nPitch KHz).

Shared access to this piezo service is controlled by first waiting on a mutual exclusion semaphore that has been pre-counted with 1 (mutual exclusion semaphores are described in the chapter Asynchronous Requests and Semaphores). The effect of the mutual exclusion semaphore, assuming the piezo is not already in use, is that the first call to p_sound will complete immediately but subsequent calls will wait until the current operation completes.

For example:

p_sound(5,320);

makes a short beep, suitable for accompanying an error notification.

.i.sound:flags,fetch;

.i.system information:sound flags,fetch;

.i.p_getsnd;	.c.::Get the sound flags (p_getsnd)

.f.INT p_getsnd(VOID);

Return the current setting of the sound flags.

The flags are described at the beginning of this section.

.i.sound:flags,set;

.i.system:sound flags,set;

.i.p_setsnd;	.c.::Set the sound flags (p_setsnd)

.f.VOID p_setsnd(INT nFlag);

Set the sound flags to nFlag.

The flags are described at the beginning of this section.

.i.system information:Series3a,sound;

.i.sound:Series3a;

.c.:Sound on the Series 3a

The Series 3a machine does not contain a piezo-electric buzzer, so all sounds are made via the loudspeaker. The Series 3a operating system does, however, contain a buzzer emulator (using the SND: device driver) that supports the sound services described in the previous section.

This does, of course, mean that the p_sound service uses more power than in the case of machines that contain a piezo-electric buzzer. Since all sounds are produced via the SND: device driver, sounds that, in other machines, use the buzzer (keyclicks, for example) are disabled while the Series 3a is playing a sound, such as an alarm.

.i.sound:files;

.i.file:sound;

.c.::Sound files

Series 3a sound files are files that contain a 32-byte header and a byte stream of digital sound. Such files normally have a .i..wve; extension.

During recording 13-bit sound (a sign bit plus 12 magnitude bits) is converted to an 8-bit data stream at 8000 bytes per second by CODEC hardware, using A-Law encoding. During playback the byte stream is sampled at 8000 bytes per second and decoded to 13-bit sound by the CODEC.

.i.header files:epoc.h;

.i.epoc.h;

In C, the file header is represented by the following struct (defined in epoc.h):

#define SignatureSize 16

#define ALawSignature "ALawSoundFile**"

typedef struct

{

TEXT Signature[SignatureSize];

UWORD Version;

ULONG Samples;

UWORD SilenceInTicks;

UWORD Repeats;

UWORD Spare[3];

} .i.structures:SndFile;

This header is written and read by the Series 3a sound services described below. The meanings of the items in the SndFile struct are:

Signature

the 16-byte (including the zero terminator)  string "ALawSoundFile**".

Version

the Series 3a sound file version number as a 4-digit hexadecimal number of the form XYYZ, where X is the major release number, YY is the minor release number and Z is normally the hexadecimal digit F. See, for example, the description of p_version.

Samples

the number of bytes following the header. This must always be size of file less the 32 bytes for the header. Dividing this by 8000 gives the duration of the sound in seconds.

SilenceInTicks

the number of system ticks of silence appended to each repeat on playback (in practice, you get at least 2 ticks between repeats).

Repeats

the number of times to repeat the sound on playback (0 and 1 are the same).

Spare

reserved for future use.

A system tick is a 1/32th of a second, equivalent to 250 samples.

The program .i.wav2wve.exe;, supplied with the SDK (it is installed to the \sibosdk\sys directory) converts .i..wav; sound files to the .wve format.

.c.::The A-Law encoding scheme

The .wve files on the Series 3a store 13 bit signed digital sound compressed into an 8 bit representation using the A-Law encoding scheme.

A-Law encoding uses a logarithmic compression to reduce the size of sound files whilst preserving the overall sound quality. The use of a logarithmic scale ensures that the low amplitiude signals (which contain most of the information in speech signals) are stored and reproduced with a minimal loss of fidelity.

The A-Law encoding scheme is simple, as can be seen from the look up tables presented in the following sections.

Encoding

.i.A-Law:encoding;The following table represents the range of possible 13 bit inputs with an x character indicating either a 1 or a zero ("don't care" states) and an s character representing the sign bit:

┌───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┐

│ s │ 0 │ 0 │ 0 │ 0 │ 0 │ 0 │ 0 │ a │ b │ c │ d │ x │

│ s │ 0 │ 0 │ 0 │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ x │

│ s │ 0 │ 0 │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ x │ x │

│ s │ 0 │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ x │ x │ x │

│ s │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ x │ x │ x │ x │

│ s │ 0 │ 0 │ 1 │ a │ b │ c │ d │ x │ x │ x │ x │ x │

│ s │ 0 │ 1 │ a │ b │ c │ d │ x │ x │ x │ x │ x │ x │

│ s │ 1 │ a │ b │ c │ d │ x │ x │ x │ x │ x │ x │ x │

└───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┘

A-Law encoding the above 13 bit inputs leads to the following range of 8 bit output values:

┌───┬───┬───┬───┬───┬───┬───┬───┐

│ s │ 0 │ 0 │ 0 │ a │ b │ c │ d │

│ s │ 0 │ 0 │ 1 │ a │ b │ c │ d │

│ s │ 0 │ 1 │ 0 │ a │ b │ c │ d │

│ s │ 0 │ 1 │ 1 │ a │ b │ c │ d │

│ s │ 1 │ 0 │ 0 │ a │ b │ c │ d │

│ s │ 1 │ 0 │ 1 │ a │ b │ c │ d │

│ s │ 1 │ 1 │ 0 │ a │ b │ c │ d │

│ s │ 1 │ 1 │ 1 │ a │ b │ c │ d │

└───┴───┴───┴───┴───┴───┴───┴───┘

Thus inputs of (1)00010111001 and (0)10000000000 give outputs of (1)1000111 and (0)1110000 respectively, where the brackets indicate the sign bit.

Decoding

.i.A-Law:decoding;The range of possible 8 bit inputs is as follows:

┌───┬───┬───┬───┬───┬───┬───┬───┐

│ s │ 0 │ 0 │ 0 │ a │ b │ c │ d │

│ s │ 0 │ 0 │ 1 │ a │ b │ c │ d │

│ s │ 0 │ 1 │ 0 │ a │ b │ c │ d │

│ s │ 0 │ 1 │ 1 │ a │ b │ c │ d │

│ s │ 1 │ 0 │ 0 │ a │ b │ c │ d │

│ s │ 1 │ 0 │ 1 │ a │ b │ c │ d │

│ s │ 1 │ 1 │ 0 │ a │ b │ c │ d │

│ s │ 1 │ 1 │ 1 │ a │ b │ c │ d │

└───┴───┴───┴───┴───┴───┴───┴───┘

Decompressing the 8 bit inputs using A-Law decoding leads to the following 13 bit outputs:

┌───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┐

│ s │ 0 │ 0 │ 0 │ 0 │ 0 │ 0 │ 0 │ a │ b │ c │ d │ 1 │

│ s │ 0 │ 0 │ 0 │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ 1 │

│ s │ 0 │ 0 │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ 1 │ 0 │

│ s │ 0 │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ 1 │ 0 │ 0 │

│ s │ 0 │ 0 │ 0 │ 1 │ a │ b │ c │ d │ 1 │ 0 │ 0 │ 0 │

│ s │ 0 │ 0 │ 1 │ a │ b │ c │ d │ 1 │ 0 │ 0 │ 0 │ 0 │

│ s │ 0 │ 1 │ a │ b │ c │ d │ 1 │ 0 │ 0 │ 0 │ 0 │ 0 │

│ s │ 1 │ a │ b │ c │ d │ 1 │ 0 │ 0 │ 0 │ 0 │ 0 │ 0 │

└───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┘

Notice that compression followed by decompression loses some of the information, as is expected.

.i.sound:Series3a,services;

.c.:Series 3a sound system services

.i.sound:record,asynchronously;

.i.asynchronous:sound,record;

.i.p_recordsounda;	.c.::Record a sound asynchronously (p_recordsounda)

VOID p_recordsounda(TEXT *name, UINT len, WORD *stat);

This function is only available in EPOC version 3.18 or later.

Asynchronously record sound to a file.

The parameter name points to a zero terminated string. This should be the filename of the file to which sound is to be recorded (any existing file is replaced). The service will fail if the file is on a Flash SSD.

The value of len specifies, in units of 2048 bytes, the maximum number of bytes to be recorded. This figure excludes the 32-byte header. Before recording starts, a file of length 32+len*2048 bytes is created (and there must actually be room on the disk for a file of that length).

While recording is taking place, the word pointed to by stat contains E_FILE_PENDING. On completion of recording, the completion status is written to *stat. The completion status will be zero if recording completed successfully, E_FILE_CANCEL if recording was terminated by a call to p_recordsoundcancel, or a (negative) error number. See the chapter Asynchronous Requests and Semaphores for a general description of asynchronous services.

The service will fail with the error E_GEN_FAIL if sound is disabled. You can only record to M: or to a RAM SSD. Although you can not record to a Flash SSD, you can copy a recorded file to a Flash SSD and play it back from there.

.i.sound:record,cancel;

.i.p_recordsoundcancel;	.c.::Cancel sound recording (p_recordsoundcancel)

VOID p_recordsoundcancel(VOID);

This function is only available in EPOC version 3.18 or later.

Cancel the recording of sound that was initiated by p_recordsounda. This service is functionally identical to p_playsoundcancel - the two services may be used interchangeably.

The sound file is truncated to the actual length that was recorded before recording was cancelled. The completion status of p_recordsounda will be E_FILE_CANCEL.

.i.sound:record,synchronously;

.i.p_recordsoundw;	.c.::Record a sound synchronously (p_recordsoundw)

INT p_recordsoundw(TEXT *name, UINT len);

This function is only available in EPOC version 3.18 or later.

Synchronously record sound to a file.

The parameter name points to a zero terminated string. This should be the filename of the file to which sound is to be recorded (any existing file is replaced). The service will fail if the file is on a Flash SSD.

The value of len specifies, in units of 2048 bytes, the maximum number of bytes to be recorded. This figure excludes the 32-byte header. Before recording starts, a file of length 32+len*2048 bytes is created (and there must actually be room on the disk for a file of that length).

The function returns when the recording is complete, and returns the completion status. This will be zero if the recording completed successfully, otherwise it is a (negative) error number.

The service will fail with the error E_GEN_FAIL if sound is disabled. You can only record to M: or to a RAM SSD. Altough you can not record to a Flash SSD, you can copy a recorded file to a Flash SSD and play it back from there.

.i.sound:play back,asynchronously;

.i.asynchronous:sound,play back;

.i.p_playsounda;	.c.:Play back a sound asynchronously (p_playsounda)

VOID p_playsounda(TEXT *name, UINT duration, UINT volume, WORD *stat);

This function is only available in EPOC version 3.18 or later.

The parameter name points to a zero terminated string. This should be either the file specification of the sound file to be played, or a * followed by just the name component of the sound file. If the string starts with a *, the extension .wve is assumed and the service automatically hunts ROM:: and the \wve directories of M:, A: and B: (in that order). Note that the Series 3a ROM:: sound files files have names sys$al01.wve, sys$al02.wve ...

.i.sound:duration;

The time that the sound file will play, in system ticks, is specified by duration. If this is shorter than the natural duration of the specified sound file then playback is truncated. If duration is negative, in addition to truncating longer files, short files are padded with trailing silence to the specified duration. If duration is zero, the file is played without truncation or padding. Note that the natural duration includes any trailing silence and number of repeats that are specified in the file header.

(The Series 3a alarm server passes a parameter of -480 to truncate or pad out to 15 seconds.)

.i.sound:loudness;

.i.loudness;

The loudness of playback is determined by volume, which may be a number between 0 and 5 inclusive, with 0 being the loudest. On the Series 3a there are only 4 actual volume levels: (0,1), 2, 3, (4,5).

While playback is taking place, the word pointed to by stat contains E_FILE_PENDING. On completion of playback, the completion status is written to *stat. The completion status will be zero if playback completed successfully, E_FILE_CANCEL if playback was terminated by a call to p_playsoundcancel, or a (negative) error number. See the chapter Asynchronous Requests and Semaphores for a general description of asynchronous services.

The service will fail with the error E_GEN_FAIL if sound is disabled.

.i.sound:play back,cancel;

.i.p_playsoundcancel;	.c.::Cancel sound playback (p_playsoundcancel)

VOID p_playsoundcancel(VOID);

This function is only available in EPOC version 3.18 or later.

Cancel the playback of sound that was initiated by p_playsounda. This service is functionally identical to p_recordsoundcancel - the two services may be used interchangeably.

After a call to p_playsoundcancel the completion status of p_playsounda will be E_FILE_CANCEL.

.i.sound:play back,synchronously;

.i.p_playsoundw;	.c.:Play back a sound synchronously (p_playsounda)

INT p_playsoundw(TEXT *name, UINT duration, UINT volume);

This function is only available in EPOC version 3.18 or later.

The parameter name points to a zero terminated string. This should be either the file specification of the sound file to be played, or a * followed by just the name component of the sound file. If the string starts with a *, the extension .wve is assumed and the service automatically hunts ROM:: and the \wve directories of M:, A: and B: (in that order). Note that the Series 3a ROM:: sound files files have names sys$al01.wve, sys$al02.wve ...

The time that the sound file will play, in system ticks, is specified by duration. If this is shorter than the natural duration of the specified sound file then playback is truncated. If duration is negative, in addition to truncating longer files, short files are padded with trailing silence to the specified duration. If duration is zero, the file is played without truncation or padding. Note that the natural duration includes any trailing silence and number of repeats that are specified in the file header.

(The Series 3a alarm server passes a parameter of -480 to truncate or pad out to 15 seconds.)

The loudness of playback is determined by volume, which may be a number between 0 and 5 inclusive, with 0 being the loudest. On the Series 3a there are only 4 actual volume levels: (0,1), 2, 3, (4,5).

The function returns when playback is complete, and returns the completion status. This will be zero if the playback completed successfully, otherwise it is a (negative) error number.

The service will fail with the error E_GEN_FAIL if sound is disabled.

.i.system:exit;

.c.:Exit the system

.i.exit to DOS;

.i.p_hwexit;	.c.::Exit to DOS (p_hwexit)

.f.VOID p_hwexit(VOID);

Exit EPOC and return to DOS (only effective on the IBM PC version of EPOC).

Calling p_hwexit has no effect on a SIBO version of EPOC.