# SIBO OPL SDK Interim Update

Version 1.0

16 September 1996

## Introduction

The purpose of this interim document is to provide essential information that will aid development of application software for the Series 3c and Siena machines. The intended audience is those developers who are using the current, version 3.00, SIBO OPL SDK.

In due course, this document will be superseded by:

·	an upgrade manual and revised software for owners of version 3.00 of the SIBO OPL SDK

·	a revised version, (V3.10) of the SIBO OPL SDK and its associated software.

The information in this document is essentially restricted to those features and/or changes that can be used with the current SIBO OPL SDK, and that the developer must be aware of when developing software for the Series 3c or Siena. The following topics are not described in this document:

·	internal hardware differences, including the changes to the expansion port socket

·	the small number of additional EPOC system services

·	the infrared device driver and related topics, such as support for IR printing

## Compatibility

The vast majority of existing software will, in principle, run on all machines in the SIBO range, provided that the display can accommodate itself to the different screen sizes.

This section describes the compatibility issues that are likely to be encountered when attempting to run existing Series 3 and Series 3a application software on the Siena and the Series 3c.

#### Programs written for the Series 3

All Series 3 programs can be expected to run without modification on both the Siena and the Series 3c. The programs will run in compatibility mode, as they do on the Series 3a.

#### Programs written for the Series 3a

Most types of program will run without modification on the Series 3c.

It is likely that most Series 3a programs will need some modification, to take account of the smaller screen size, before they will run on the Siena. Menus and dialogs in Series 3a programs will, in general, be too wide and will need to be reorganised and/or reworded. Applications that use a sophisticated layout in their display are likely to need extensive modification before they can be used on the Siena.

## Screen and keyboard

This section describes the major changes and additions that have been made with regard to the screen and keyboard of the Siena and Series 3c machines.

### Screen properties

The true screen and pixel dimensions of the various LCD screens on all SIBO machines are as follows:

| Machine type | Screen (pixels) | Pixel pitch (mm) | Pixel size (mm) | Screen size (cm) | Screen size (in) |
| --- | --- | --- | --- | --- | --- |
| HC | 160x80 | 0.34x0.43 | 0.31x0.40 | 5.44x3.44 | 2.14x1.35 |
| S3 | 240x80 | 0.385x0.43 | 0.355x0.40 | 9.24x3.44 | 3.64x1.35 |
| Workabout | 240x100 | 0.26x0.30 | 0.23x0.27 | 6.24x3.00 | 2.45x1.18 |
| Siena | 240x160 | 0.25x0.25 | 0.23x0.23 | 6.00x4.00 | 2.36x1.57 |
| S3a | 480x160 | 0.259x0.259 | 0.20x0.20 | 12.477x4.156 | 4.915x1.637 |
| S3c | 480x160 | 0.26x0.26 | 0.20x0.20 | 12.478x4.158 | 4.915x1.637 |
| MC200 | 640x200 | 0.33x0.33 | 0.30x0.30 | 21.12x6.60 | 8.31x2.60 |
| MC400 | 640x400 | 0.33x0.33 | 0.30x0.30 | 21.12x13.20 | 8.31x5.20 |

In the above table, the horizontal measure is shown before the vertical measure. The pixel pitch measures the horizontal and vertical distance between the same points on adjacent pixels. The difference between the pixel size and the pixel pitch gives the gap between pixels.

The Series 3c and the Siena screens support the use of grey in exactly the same way as for the Series 3a.

### Keyboard

Except where stated below, the keycodes produced on the Series 3c and the Siena are identical to those produced on the Series 3a.

The Siena has a fn modifier key that produces an additional modifier (hexadecimal $80) that may be returned by KMOD. This value is the same as that for mouse button down, previously only used on the MC range of machines.

The following table lists the additional and/or modified key codes produced by the special keys on the Siena and the Series 3c:

| $9 or '\t' | Produced by tab, with or without shift or ctrl. On the HC and MC only, the psion key adds $200. On the Workabout, the psion key (but not shift+psion) converts the keypress to $108 (Task). On the Series 3c, the psion key (but not shift+psion) converts the keypress to $142 (IR Link). |
| --- | --- |
| $131 to $139 | On the Series 3 and Series 3a, key codes $131 to $138 are produced by the eight membrane keys (also called application keys) in order, from left to right. The Series 3c has a ninth membrane key, on the extreme right, that produces $139. |
| $140 | On the Siena only, this key code is produced by the IR receive key. |
| $141 | On the Siena only, this key code is produced by the IR send key. |
| $01B | On the Siena only, this key code is produced by the on/ce key, (calculator clear). |
| $150 | On the Siena only, this key code is produced by fn + the ‘3’ key on the numeric keypad, (calculator memory clear). |
| $151 | On the Siena only, this key code is produced by fn + the ‘2’ key on the numeric keypad, (calculator memory recall). |
| $152 | On the Siena only, this key code is produced by fn + the ‘-’ key on the numeric keypad, (calculator memory minus). |
| $153 | On the Siena only, this key code is produced by fn + the ‘+’ key on the numeric keypad, (calculator memory plus). |
| $154 | On the Siena only, this key code is produced by fn + the ‘.’ key on the numeric keypad, (calculator change sign). |
| $155 | On the Siena only, this key code is produced by fn + the ‘=’ key on the numeric keypad, (calculator percent). |
| $156 | On the Siena only, this key code is produced by the ‘.’ key on the numeric keypad, (calculator decimal point). |
| $158 | On the Siena only, this key code is produced by fn + the ‘1’ key on the numeric keypad, (calculator memory input). |

## Windows, menus and dialogs

There is no significant change to the OPL language on either the Series 3c or the Siena.

The Siena’s 240x160 screen is half the width of the Series 3a and Series 3c screens. In consequence, great care has to be taken to ensure that lines of text do not exceed the available width. This restriction particularly affects the amount of text that can be displayed in the menu bar and in dialog items.

In general, programming for the Series 3c is identical to programming for the Series 3a.

### Status windows

#### Siena

Only a narrow status window is available on the Siena. A smaller font is used and the diamond list has been reorganised, compared with that of the Series 3a, to maximise the space for displaying text. Despite these changes, only four characters of the application name and up to five characters of each diamond list item can be displayed.

Diamond list text items that contain more than five characters are automatically truncated. Since such truncation may occur at an unsuitable point in the text, you should consider supplying truncated versions of the diamond text for use in the status window, such as the four-character abbreviations for ‘Normal’ and ‘Outline’ shown in the above illustration.

#### Series 3c

Status windows on the Series 3c are functionally identical to those on the Series 3a. Some minor cosmetic changes have been made, including a new analog clock design for the wide status window.

### System dialogs

#### Siena

The Siena does not support tone dialling.

The content of the various Print dialogs has been rearranged.

#### Series 3c

The Link dialog offers a selection of Baud rates up to a maximum of 57600, compared with the maximum of 19200 on the Series 3a. Two additional baud rates may be selected:

Baud rate

38400

57600

Shift-Tab (instead of Psion-Tab) displays the current path in an information message (Psion-Tab is now used to instigate IR file or record transfer).

### Dialogs

#### Siena

Dialogs are displayed in an 11 point font, with thinner borders, as on the Workabout. Up to eleven lines of text, including the dialog title, can be displayed in a Siena dialog.

Text items in a dCHOICE choice list are automatically truncated if they are too long to be displayed.

#### Series 3c

Dialog borders have been restyled, but the basic geometry is unchanged. There is no difference in the programming interface.

### File lists

#### Siena

File lists no longer display the time and date for each file. Pressing the space bar shows the time and date for the currently selected file.

### Calendar windows

#### Siena

The Siena can only display the data for a single month.

### Menu bars and pull-down menus

#### Siena

The menu bar and pull-down menu borders are thinner than those on the Series 3a , being similar to those used for the Workabout, and the minimum spacing between menu bar items has been reduced. Despite these changes, a typical Siena menu bar can not contain more than, say, five menu titles plus a diamond menu, and the text is restricted to around 26 characters in total. Siena applications with many menu options will, in general, need to make greater use of submenus than equivalent applications running on the Series 3a or Series 3c.  However, the Siena screen’s size is unlikely to restrict individual pull-down menus, which may contain up to nine items.

#### Series 3c

Menu borders have been restyled, but the basic geometry is unchanged. There is no difference in the programming interface.