VgaSuite (Version 1.17)

Overview

VgaSuite, developed by Osirix Design Ltd,  is a package which enables the user to connect any standard external PC monitor through the Voyager VGA PCMCIA card for the display of slides and/or 'current screen' images.

Three refresh rates are provided:  Slow, Medium and Fast. These allow for the display of both Still and Moving images. The slide-show utility displays BMP and PCX files. Up to 256 colours and resolutions of 640x480 and 800x600 are supported.

The latest version (version 1.17) now includes real slide-show support such that either a continuous slide show can be shown (automatic mode) or the next slide is shown when a certain key is pressed (manual mode).

A must for all presentations!

Installation Guide

###### Method 1

- Download VgaSuite.sis onto your hard-disk.

- Connect your Psion to your desktop (or laptop) via PsiWin.

- Double-click on the VgaSuite.sis file.

- Follow the on-screen instructions.

###### Method 2

- Download VgaSuite.sis onto a Flash card.

- Insert the Flash card into your Psion.

- Double-click on VgaSuite.sis file in D: or E: depending on where the flash card is.

- Follow the on-screen instructions.

#### Use of the VgaSuite Software

PLEASE NOTE – the driver software has been developed for use ONLY with the following VGA card, manufactured by the Colorgraphic Communications Corp:-

Voyager VGA PCMCIA Bus Super VGA Adapter

- Install the card into the netBook/Series 7 PCMCIA slot and connect the display device to it. The display device could be a monitor or a projector. Power-up and switch on the display device.

- There are two methods to make use of the card in order to output to the display :-

##### Slide VGA

The Slide VGA utility allows you to display single BMP or PCX format files, or a continuous slide show. For the format of how a slide show is displayed, see the example in \slides\slide.shw. Run Slide VGA as follows:

    - From within the Extras toolbar run Slide VGA.

    - From its main menu, go to Tools-Settings

    - Set Slide show to Automatic or Manual (see below for details) and set Video Card to On

    - Set the Update Rate to either Slow, Medium or Fast and tap OK

    - The netBook/Series 7 display should now be output to the chosen display device.

    - From the main menu, go to File-Open image to display a single image, or go to File-Run slides to run a slide show.

    - Use the green EPOC sign icon to switch between other applications

    - To turn the output off, set Video Card to OFF within the Slide Vga application

Real slide show support. If you enable "Manual" from "Preferences->Slide show" then the application will wait after displaying each picture for the following keys :-

SPACE, left arrow, up arrow - Next slide.

Down arrow, right arrow - Previous slide.

The name of the slide being displayed will be displayed in the top right corner of the screen. The user can go back to the timer driven slide show by selecting "Automatic" (default) from "Preferences->Slide show". You can switch between modes at any point during the slide show.

Full Toolbar support. The Toolbar now contains shortcuts to the following menu options: "Open image", "Run slides", "Stop slides". It also has buttons for "Next Slide" and "Previous Slide". However these buttons are only active when a manual slide show has been loaded. The Toolbar also contains the battery and clock to mimic the built in applications.

The time the picture is displayed can be made optional. If the time is omitted then a default of 15 seconds will be used when the slide show is run in "Automatic" mode.

##### HotKey VGA

The HotKey VGA application allows you to set-up HotKeys to access the application. The default HotKey is Shift_Fn_A with the use of Application Key 1. These default values can be changed from within the HotKeys tab in the application window. Run HotKey VGA as follows:

- From within the Extras toolbar run HotKey VGA

- Set Display to ON

- Set the Update Rate to either Slow, Medium or Fast and tap OK.

- The netBook/Series 7 display should now be output to the chosen device.

- To turn the output off, set Display to OFF from within the HotKey VGA application.