Translated from the German Original

PsiNMEA v2.11b, release 10.05.2000 
Author: Torsten tree brook 
Programming language:  OPL 
Platform: EPOC 
Test equipment:  Psion series  5 Classic (works with 5MX), Garmin 12 GPS receiver
Requirements:  OPX modules SystInfo, RMRAlarm, nDirNav 
Short description 
PsiNMEA carries out the following main tasks: 
 Reading and analyzing  various data from NMEA compatible devices. 
 Reading and analyzing  specific NMEA data from a  GPS receiver. 
 Storing and analyzing of the data using the Garmin Protocol. 
 Administration of Waypoints, routes and Tracklogs including sending and receiving via Garmin protocol. 
 Different GPS specific functions, like showing the position on maps or  analysis of Tracklogs. 
 Special functions, like Datum conversion, distance computations or solar eclipse computations

PsiNMEA is Freeware and open SOURCE. See also: 
http://www.baumbach - web.de 
The use in commercial products without written permission is 
strictly forbidden. The entire copyright is with the author of PsiNMEA! 
The menu 
Program | Protocol | NMEA 
 
 Provides a connection over the serial interface and activates all NMEA functions of  equipment attached by PsiNMEA.
 The adjustment of the serial interface values specified in the NMEA protocol, can however if necessary be  changed under 
Program> Settings> Serial port … -> NMEA … 
The serial interface may not be used by another program (or  
by the system) .The same applies to the IrdA interface. 
The attached equipment must  be adjusted to read or write in NMEA format. 
(In the GPS12 the adjustment reads NMEA/NMEA or NONE/NMEA ).

Program | Protocol | GRMN 
 Provides a connection over the serial interface and activates all the GRMN functions of equipment attached by PsiNMEA.
 The adjustments of the serial interface correspond to the values specified in GRMN protocol, and can be made in
Program> Settings> Serial port … -> GRMN… if a change is necessary. The serial interface may not be used by any other program (orby the system) . The same applies to the IrdA interface. 
The attached equipment must be adjusted to GRMN. 
(In the GPS12 the adjustment reads GRMN/GRMN ). 
Program | Protocol | Disconnect 
 An existing  connection to  NMEA or GRMN  compatible equipment is closed. All special NMEA or GRMN  functions of NMEA are deactivated. The interfaces of the  EPOC equipment can be now used  by other devices. 
Program | Settings | Program … 
 A dialogue for the definition of fundamental program adjustments is  opened. The dialog fields have the following meaning: 
 Establish connection - If  start NMEA or   start GRMN is selected, then at the start of  the program PsiNMEA a connection is made over the serial interface with that  appropriate protocol .
 Fast  file  dialogue  If this switch is activated, then  the nDirNav OPX modules are used  instead of the standard selection of files . These make possible a faster search with long listings.
 Show system folder  This switch is relevant only if the standard  Selection of files dialogue is to be used. In this case the user can  select whether in the listing tree the EPOC system listing is available. 
 The deactivation of this switch generally causes a faster  selection of files. 
Coordinate fields order  selects the order of input and output of Coordinates of  Latitude or Longitude i.e.  Lat-Long or Long-Lat
Negative latitude/longitude sets whether North, South,East or West are shown /input as negative.
 Allow wpt duplicates into databases  If this switch is activated, then is it is possible in a Waypoint data base file (*.wpt) for several waypoints  to have the same name. If set to off repeated Input of the same Waypoint ID  will overwrite the previous record.
 Note! When transfering waypoints from GPS equipment overwriting  of duplicates will occur. 
Allowed map datum convertion error -This adjustment affects the Map datum converter . The computation of the Latitude/Longitude/ Altitude data from the geocentric ECEF system is complicated and cannot take place with 100 percent accuracy. 
If one selects the value 0 here ”, then a simple procedure is used, suchthat an altitude accuracy within the centimeter range for heights< 1000km  is achieved.
For values >0 ”an iterative procedure is used that with a maximum of  50 iterations, tries too reach the indicated accuracy. Generally the adjustment 0 is better ”. 
Program | Settings | Output… 
A dialogue for the definition of the output formats is opened. The user can select  units and representation of different sizes of (time/distances … )

Program | Settings | NMEA socket… 
 Different adjustments of the NMEA Parsers can be selected: 
Proprietory data  -The manufacturer can be selected, 
If (ignore) is set, then all proprietory data is ignored. 
Viewing  format  Here the quantity of the NMEA sentences can be reduced .This adjustment has no influence on the Parser. 
CRC error  The actions  on errors is specified. 
 Prompt on …  The user can specify actions for different events 
(Check accumulated error, read error, internal syntax error, unknown quantity ) It selects a NMEA sentence,and  whether the Parser is to ignore this error, or whether to warn the user
 With the Button New … additional manufacturers can be added to the list one at a time. This function is interesting only for developers, who wish to amend the  source code of PsiNMEA  
Program | Settings | GRMN socket … 
 At the present time no adjustments for the GARMIN protocol can be  selected 

Program | Settings | Serial port | NMEA … 
 This is for the adjustments of the serial interface for the NMEA protocol. With the Button Default these default values are set up for  NMEA Standard definitions 
Program | Settings | Serial port | GRMN… 
 This is for the adjustments of the serial interface for the GRMN protocol . With the Button Default the default GRM values are set up .
The additional adjustment WAIT for host COMMAND before sending

Before sending waypoints or routes to a GPS  a send command from the GPS is required.

. (In the GPS 12 the adjustments read  GRMN/GRMN read WPT or GRMN/GRMN read RTE). In the other case the transfer without  waiting a command to the equipment is sent. 
(In the GPS 12 the adjustments read then GRMN/GRMN host waits …). 
NMEA | About DEVICE … 
 Information about the attached NMEA equipment is read and indicated (equipment, type of device and software -Version). 
 Additionally the proprietory record formats can be queried. 
NMEA | Set of listener ID … 
 The type of device can be set that  the Psion  is to emulate when sending NMEA inquiries to the attached equipment  (See also NMEA> 
READ special … - > Query … ) 
NMEA | Monitor | sentences … 
 Data arriving over the serial interface is shown as NMEA sentences . This setting  will determine for how long the sentences are monitored.   Monitor mode to be terminated  - (adjustment 0 = for an unlimited period) and whether  the information is to be written into a text file. 
 The monitor mode can be stopped at any time by pressing ESC

NMEA | Monitor | parsed data …

Data arriving over the serial interface is read as sequential  NMEA sentences and the data/the information contained in them shown. The user can select whether to pause between the individual sentences . In this case  after each sentence press ENTER to
continue. The monitor mode can at any time be discontinued by pressing ESC 
NMEA | Monitor | from file … 
 Similar NMEA> monitor> parsed datas … , however the NMEA sentences are  read from text files which can be selected. 
NMEA | Terminal 
 By command line - NMEA sentences can sent to the attached equipment .
 The command line is terminated by inputing  Q .
 With the input C the check  mode can be switched off/. 
 With this mode switched on the check sum is attached to each sentence.

NMEA | READ special | DATE & Time 
 
 Current time and date is determined from the NMEA data  and  shown. In addition the clock of the EPOC equipment  is synchronizable.

NMEA | READ special | GPS location 
The Latitude/Longitude coordinates of the current position are indicated.  
Map datum corresponds to the settings on the GPS equipment.

NMEA | READ special | userdefined… 
Sequentially selected data of a selected NMEA sentence is shown.
the user can select whether to display continuosly   or whether a key press is waited for after each update.
NMEA | READ special | Query … 
After the selection of specific  NMEA sentences these are  obtained from the GPS. 

NMEA | Statistics … 
Sequentially selected data from a selected NMEA sentence is shown.
.
For each individual data field a statistical  evaluation is possible. It can in each case show ` only view ' (no evaluation), ` AVERAGE ’(Average value), ` deviation ' (Deviation of kind of condition), ` Garmin/max' (Extreme values) and ` all' (Average value, hr. deviation and interval) to be selected. 
The printing of the data    in a text file possible. In addition the user can select if the data is to be shown on a continuous basis as it is updated updates (Stepping = automatical) or whether after each display a key press is awaited. (Stepping=manual). 
The rate of automatic stepping can be defined.

Garmin | About DEVICE … 
  Information about the attached equipment can be displayed. 
 The menu entry `Garmin ' is present only if Program> Protocol 
- > GRMN gps protocol is off.

Garmin | READ DATE & time …

GPS  time and - date can be displayed.
 The menu entry `Garmin ' is present only if also Program> Protocol 
- > GRMN   protocol is off
Garmin | READ GPSt location … 
 The current GPS coordinates can be displayed
 The menu entry `Garmin ' is present only if also Program> Protocol 
- > GRMN  Garmin protocol is off.
Garmin | READ PVT datas … 
 The GPS time,  coordinates,  and Speed can be displayed.
 The menu entry `Garmin ' is present only if also Program> Protocol 
- > GRMN  protocol is off.
Garmin | Power off … 
 The attached equipment is switched off. 
 The menu entry `Garmin ' is present only if also Program> Protocol 
- > GRMN  protocol  is off.
GPS | Location on map | Current location … 
 After the selection of a map and its its display on the Psion    the current position is indicated by means of a cursor.  This is sequentially updated. 
 See also: Map mode 
GPS | Location on map | Entered location … 
After the input of coordinates, the selection of a map and its display on the Psion 
the entered position is indicated by means of a cursor. 
 See also: Coordinate input, Map mode 
GPS | Tracking … 
 See Databases> TRACK> Tracking … 
Databases | Map registry | ADD map … 
A new map can be added to  the PsiNMEA maps with this function
Register.
After the selection the user enters a short description for the MBM file. In all dialogues which require the selection of a map this description is used.instead of the file name .
A new map must be calibrated . The coordinates of two points on the map   must be know and entered. All maps using Latitude Longitude coordinates must  meet the right- angled, linear projection. 
The calibration can take place in two different ways: 
Button Calibrate : The map on the screen is used The two points of calibration must in each case be selected  with the stylus and  arrow keys and ENTER.  (Reference: The further the two points lie apart, 
the more exactly the position on this map can be indicated later. It is thus recommended, for example one point is in the left, upper corner and one in the right lower corner of the map.) 
Button Calibration from ANOTHER map … : If a map of the same size and covering the area is registered then the calibration data can be used for the new map.

Databases | Map registry | Edit map … 
For maps already registered  the calibration data can be changed. A change of the file directory is possible. See in addition Databases> map registry> checks … Databases | Map registry | Check … 
 All registered maps are checked for consistent calibration data . 
 In addition a check takes place whether the associated MBM files still  exist. If   not the current location can be entered if available. 
 Note: If the calibration data of a map is inconsistent and therefore is changed, this function must be called again because the message ` All registered maps of are okay. ' applies then only to those  unchanged maps. 
Databases | Map registry | ADD world map … 
 In order to make possible immediate  use of PsiNMEA the map of the world used by K.Millican’s programs is needed. Before this map can be used, it must be transferred to the map Registry together with calibration data. However the calibration data for this map is known by PsiNMEA so no user calibration is needed. 
Databases | Map datum | ADD/Edit… 
 PsiNMEA possesses a data base of approximately 220  map datum and  their  Parameters (ellipsoid + 3 parameter for simple ECEF SHIFTs). 
 This data base can be changed by the user at will. In addition is 
 however fundamental knowledge of map datum and map datum Conversions is needed.. 
 For a new map datum you need: 
 Name  short name of the map datum 
 Description short description, with which pre-defined 
 Map datum   the ellipsoid name 
 Region   of validity of the map datum 
 Semi major axis Large shaft section ellipsoids in meters 
 Flattening  Flattening ellipsoids 
 GPS-ID  Currently unused ( - 1) 
 X - /Y - / Z -SHIFT  Shift of the ECEF system relative 
 WGS84 
 X - /Y - / Z -rotation Rotation of the ECEF system concerning WGS84 
 (only with accurate helmet blank transformation) 
 Scale parameter Scaling of the ECEF system bezgl. WGS84 
 (only with accurate helmet blank transformation) 
 Direction  Direction of the indicated parameters 
 ( WGS84> date or Date> WGS84) 
 In order to simplify the input of the ellipsoid parameters the Button Ellipsoid… gives access to a data base of nearly all important Ellipsoids . 
 At present PsiNMEA does not convert coordinates on line.

Databases | Waypoints | ADD/Edit … 
 In Waypoint data bases (*.wpt - files) the coordinates 
 Height over the sea level, an Waypoint ID (max. 6 indications), a description and an symbol ID arestored. Whether a Waypoint ID can occur several times within a data base is specified in Program> Settings> Program 
 After selection of this menu option first  a dialogue  indicates the last Waypoint data base used . This can be changed by the Button CHANGE file … The production of a new, empty Data base usesthe Button Create new …. 
With the Button Edit waypoint  one reaches the detailed Waypoint list of the selected data base. There one can add, edit or delete waypoints. See also: Coordinate input 
Databases | Waypoints | Send… 
 After the selection of the Waypoint data base all Waypoints in this file can be transferred to the attached equipment using the Garmin protocol. 
 Note! Waypoints with the same ID in the Garmin equipment are overwritten unless the manual states otherwise.  
 Note! Symbols are not transferred in the current version yet. 
Databases | Waypoints | READ … 
 After the selection of the Waypoint data base all Waypoints  are read into the selected database using Garmin protocols of the attached equipment. Depending on Program> Settings> Program … fixed adjustment , the new data is added as  Waypoints without test of duplicates or all duplicates are overwritten 
Note! Symbols are not read in the current version yet. 
Databases | Waypoints | Export … 
 This function exports  a data base of the waypoint data into a readable text file. The transfer takes place in a table format. Every  Line contains the data of  a waypoint. The allocation of the data ( ID , Latitude, Longitude, Altitude, description) can be specified on the columns in the first input dialogue. The transfer of Altitude and Description can be also suppressed (adjustment hide ). The user can select
 which column separation (SPACE/TAB/ COMMA ) is to be used. The output format for Latitude/Longitude data export can be freely specified  independently of the global program adjustment .
In the second input dialogue is (new) the file name can be provided . With this function  waypoint lists can be exported in a text file and also be imported into a route database.
Databases | Waypoints | Import … 
 This function allows the reading in of waypoint data from a readable 
 Text file. The structure of this text file under Databases> Waypoints 
- > Export … must correspond to described format. After the selection of the 
Text file, the file can be mapped to the specified format.Under Program> 
 Settings> Program … fixed adjustment , the new data will become 
 Waypoints without test and duplicates added or all duplicates are overwritten.

Databases | Waypoints | PsiNMEA v2.00 … 
The structure of the waypoint data bases has changed after version 2.00 . Data bases with a PsiNMEA version <  =2.00 are not error free in later versions.  
However it is possible  to convert “old ”data bases into the new format with this menu entry. 
Databases | Routes | New one/Edit… 
In route data bases (*.rte - files) the coordinates are 
 Height over the sea level, an Waypoint ID (max. 6 indications) and an symbol ID . 
 After selection of this menu  option it is possible to change Button CHANGE file or create a new file using the Button Create new … 
The Button Edit waypoint  shows the detailed Waypoint list of the selected data base. Here one can add, edit, delete or change the order off waypoints.. 
See also: Coordinate input 
Databases | Routes | Analysis/Draw … 
After the selection of a route data base the user can analyse routes, distances etc and show thhem on a map.

PAGE … more data can be shown .  In addition the altitude of the route   can be shown in a diagram (Button Altitude diagram … ). 
Databases | Routes | Send… 
After the selection of a route data base and a route number 
becomes the route of the attached Garmin belonging to to this number 
Equipment by the route data of the route data base replaces. 
Note! Symbols are not transferred in the current version yet. 
Databases | Routes | READ … 
After the selection of a route data base and a route number the route can be sent to an attached Garmin GPS.Any route with the same number will be overwritten.
. 
Note! Symbols are not read in the current version yet. 
Databases | Routes | Export … 
Similar Databases> Waypoints> export… 
With this function in a text file of a route can be exported also routes can be imported from a text file. with
Databases | Routes | Import … 
Similar Databases> Waypoints> import… 
Databases | Routes | Streetplaner 
 With this function a route in Streetplaner overlay format can  be created .  
 . This function is © tungsten Mueller 
Databases | Routes | PsiNMEA v2.00 … 
The structure of the route data bases changed after the version 2.00 
 Data bases with a PsiNMEA version <  =2.00 are no longer error free.
 It is however possible with this menu entry to convert “old ”data bases into the new format .

Databases | TRACK | Tracking … 
After the input of a short description 
 TRACK Logs and the time interval between two position measurements  in  the map mode are recorded.
 The menu entry Stop and save … the recording can at any time be ended
 The maximum possible number of storable points is currently  512. If this limit is reached, Recording is interrupted for a short time and a more complex, adaptive Segmenting algorithm removes  irrelevant measuring points. 
Alternatively TRACK Logs can also be written with most GPS devices . These can then be transferred later to the PSION using Garmin protocols  (Databases> TRACK> READ). 
Databases | TRACK | Analysis/Draw … 
Similar Databases> Routes> analysis/Draw … 
Additionally TRACK evaluation is available  showing the TRACK in a distance map and different speed diagrams. 
Databases | TRACK | READ … 
After the input (new) of a file name (*.trk), a short Description of the TRACK Log it is possible to reduce the number of track points by selecting what is read by the Psion.

Databases | TRACK | Streetplaner 
 With this function a TRACK log in the Streetplaner overlay format can be created.This is . © tungsten Mueller 
Databases | TRACK | ADD ton of route … 
After the selection of a TRACK data base and a route data base 
 all measuring points of the TRACK are added as new Waypoints to the route .They receive thereby the reference no. FIX000, FIX001, … . A  symbol ID for these new waypoints can be specified. 
More | Unit converter … 
Here different physical units can be converted 

More | Map date converter … 
Here Latitude/Longitude coordinates can be converted to another 
Map datum or into the associated ECEF system . 
See  Databases> map date> ADD/Edit and Program> Settings> Program . for issues about accuracy of data.
More | These distance calculator … 
This calculates the spherical distance between two points on the earth's surface  after their coordinates and a uniform Height over the path is entered. 
More | Sun eclipse | Local circumstances | current location … 
 PsiNMEA has the data for the total sun eclipses of 11.08.1999 and 21.06.2001. After the selection of the associated 
Data files (19990811.dat or 20010621.dat)  
 the most important data (Distance to the central path, duration of the Totalität, … )is computed for the current Position (only with activated NMEA protocol)

More | Sun eclipse | Local circumstances | entered location … 
Similar More> Sun eclipse> local ones circ. - > current location … , for any, position which can be entered. 
More | Sun eclipse | Draw path … 
After the selection to the desired solar eclipse Data files  and the selection of a map the path of the totality will be shown.
More | Backlight 
It can be in/switched off or on to give continuous background lighting. 
More | Auto off 
The autooff mode of the Psion can be activated . 
More | Task manager … 
The window of the EPOC task manager is opened.

Coordinate input 
PsiNMEA possesses a Parser for Latitude and Longitude inputs. Therefore the Format of the input only slightly prescribed. The following example inputs are permitted and possess same value: 
- 12 ° 00' 20.8 - 12 ° 00' 20.8 ”  12 ° 00' 20,8 ”S 
- 12 ° 0.34667 - 12 ° 0.34667' 12 ° 0.34667' S 
- 12.0057778 - 12.0057778 °  12.0057778 ° S 
Likewise the indications can in nearly all cases ° / ’/ ” can be replaced by  d/m/s . Note: In this example it was accepted that under Program> Settings> 
Program the meaning of a negative Latitude value is South 
In all dialogues, in which the input of coordinates is required   then
… (Ctrl+W) enables a Waypoint from a Waypoint data base or the World data base to be used.
If NMEA protocol are operational at the time of the coordinate inquiry, (GPS is connected )then  (Ctrl+C) the current GPS coordinates can be taken into the Input fields to be taken over. 
Map mode 
In the map mode (announcement of the current or entered position; Tracking that 
Position; Indicate a solar eclipse path) is available the following menu. 
(The entries can vary depending upon task slightly.) 
… ASSISTANCE LATE SUPPLEMENTAL … ..

Key contraction (CTRL Shortcuts) 
CTRL + A NMEA | About DEVICE …  or Garmin | About DEVICE … 
CTRL + B More | Backlight 
CTRL + C GPS | Location on map | Current 
CTRL + D Program | Protocol | Disconnect 
CTRL + E Program | Exit 
CTRL + G Program | Protocol | GRMN 
CTRL + I Databases | Waypoints | Send 
CTRL + J Databases | Waypoints | READ 
CTRL + K Databases | TRACK | READ 
CTRL + L Databases | Routes | Analysis/Draw 
CTRL + M NMEA | Monitor | sentences 
CTRL + N Program | Protocol | NMEA 
CTRL + O Program | Settings | Output 
CTRL + P Program | Settings | Program 
CTRL + Q Databases | Map of date | ADD/Edit 
CTRL + R Databases | Routes | ADD/Edit 
CTRL + S Databases | TRACK | Analysis/Draw 
CTRL + T GPS | Location on map | Tracking  and/or. Databases | TRACK | Tracking 
CTRL + U  More | Car off 
CTRL + W Databases | Waypoints | ADD/Edit 
CTRL + X Databases | Waypoints | Export 
CTRL + Y Databases | Waypoints | Import 
* Only in the NMEA protocol mode 
** Only in the Garmin protocol mode 
*** Only if no protocol are active 
* ** 
*/** 
*/** 
*/*** 
** 
** 
** 
* 
**/*** 
*/** 
*/** 
.

Key contraction (SHIFT CTRL Shortcuts) 
Shift+Ctrl + C More | Unit converter 
Shift+Ctrl + D NMEA | READ special | Date&Time or Garmin | READ Date&Time 
Shift+Ctrl + E GPS | Location on map | Duck-talk 
Shift+Ctrl + F NMEA | Monitor | from file 
Shift+Ctrl + G Program | Settings | Serial port | GRMN 
Shift+Ctrl + I Databases | Routes | Send 
Shift+Ctrl + J Databases | Routes | READ 
Shift+Ctrl + L NMEA | Set of listener ID 
Shift+Ctrl + M NMEA | Monitor | parsed datas 
Shift+Ctrl + N Program | Settings | Serial port | NMEA 
Shift+Ctrl + P NMEA | READ special | GPS location or Garmin | READ GPS location 
Shift+Ctrl + Q More | Map datum converter 
Shift+Ctrl + R Databases | TRACK | ADD ton of route 
Shift+Ctrl + S NMEA | Statistics 
Shift+Ctrl + T NMEA | Terminal 
Shift+Ctrl + V Garmin | READ PVT datas 
Shift+Ctrl + X Databases | Routes | Export 
Shift+Ctrl + Y Databases | Routes | Import 
Shift+Ctrl + Z More | These tanks calculator 
* Only in the NMEA protocol mode 
** Only in the Garmin protocol mode 
*** Only if no protocol are active 
© May 2000, Torsten tree brook 
All data are without guarantee! No adhesion for any damage is taken over, those 
by the use of PsiNMEA or by incorrect data in this documentation 
develop. No requirements result opposite the author of PsiNMEA.