# How to Setup PassThru

This procedure assumes that you are running Windows95 or '98.

## Step 1 - Determine your modem COM port settings (Windows)

- Look at your System properties from the Windows control panel.

- In the Device Manager section, locate your modem and find out its COM port by looking at its properties.

- Look at the COM port properties for the port attached to your modem. Make a note of this COM port number and its IRQ (Interrupt Request) number. The IRQ number is in the 'Resources' section.

## Step 2 - Identify the COM port for attachment to the Psion (Windows)

- Look at the other COM ports available in the Device Manager section and note down their IRQ numbers.

- If one of these other COM ports uses the same IRQ number as the port attached to the modem, then this is the one to avoid using

- Check that your Psion cable is attached to a port with a different IRQ number to your modem port.

## Step 3 - Check for port usage. (Windows)

- Examine the taskbar to check that 'Copy Anywhere' and 'PsiWin' are not running. If they are then right click on each with the mouse and select 'Close'.

- Make sure that any other software that might hog a COM port is shut down. (Typically fax receiving software or Windows Internet Explorer and/or EMail programs)

## Step 4 - Run PassThru and Setup Options (Windows)

- Run the PassThru application and switch to full screen (press Alt-Enter to toggle between full-screen and windowed modes)

- Press the spacebar to setup the ports.

- Enter the details for the COM port used by the Psion (*).

- Enter the details for the COM port used by your modem (*).

- * In both cases enter a suitable baud rate/speed - this is typically double your maximum modem connection speed but you must pick a speed supported by the Psion, ie. 9600, 19200, 38400, 57600, or 115200. The following are recommended :-
14.4 modem, use 19200
28.8 modem use 38400
33.6 modem use 57600
56K modem use 115200 (assuming your PC hardware is up to it !)

- IF YOU WISH TO ATTEMPT FAX SEND/RECEIVE THEN YOU MUST SET THE SPEED TO 19200 BAUD BECAUSE MESSAGE SUITE IGNORES YOUR MODEM SETTINGS FOR FAXING.

- * Make sure that you enter the correct IRQ numbers for both ports.

## Step 5 - Set up the Psion (Series 5)

- Make sure that the communications link is switched off.

- Go to the Control Panel and tap the 'Modems' option.

- Select 'Hayes Compatible' as the type.

- Tap 'Edit' and alter the speed to match the rate used by PassThru.

- Go to the Options tab and make sure that the ‘Init’ string is suitable for your modem (you will need to look at your modem handbook for this.

- Go to the handshaking tab and ensure that ‘Flow Control’ is set to Hardware ‘(RTS/CTS)’ and that the two option boxes are clear.

## Step 6 - Test the PassThru Program (Series 5)

- Run the Psion COMMS application.

- Tap the 'Setup' button in the Toolbar and set the speed to match that used by PassThru.

- Type a Hayes command followed by the <return> key. Typical Hayes commands are, eg. AT&v ATI?

- If you can see characters as you type and you get a reply followed by 'OK' then everything is working properly.

- If you can't, try typing AT&F followed by <return> to reset the modem.

## Step 7 - Set up your Psion Internet Connection. (Series 5)

- Refer to your Web manual on the PsiWin disk and your PC's dial-up connection settings.

## Other things that can go wrong (Windows)

- Some programs seem to incompletely release the COM ports. It can be useful to restart Windows, or even run PassThru from DOS mode. A less drastic option that usually works is to escape from PassThru and run it again - this seems to clear it.

- If your PC UARTs are below 16550A then you may not be able to use the fastest baud rates. This problem is made worse by running the program on a slow (ie. Below Pentium) PC

- If you have other programs running, this can reduce performance.

- For some reason Psion Email connects better when you access it after making a connection with Web. This seems to be the case whether I use PassThru or a direct modem connection - however this may just be a feature of my ISP.

- Some ISPs are unusable with the current version of Psion Message Suite. I believe this is due to their need to receive your login data in an encrypted form that is currently unsupported by Psion.

Kevin Millican

(kmillican@compuserve.com)