HVACalc V1.1

Psychrometric, and velocity pressure calculator.

Instructions on Use:

Double click on the HVACalc icon in the Eclipse Program Manager Group.

Setting Temperatures

Use the scroll bars to change the dry and wet bulb temperature values. By clicking the scroll bar buttons, the temperatures change in increments of  0.1 Celsius. By clicking between the buttons and the scroll bar position marker, the temperatures change in increments of 1 Celsius. It is also possible to drag the scroll bar position markers with the mouse for large temperature changes.

Other information shown

The following data is updated when the temperatures are changed and the mouse buttons are released.

Saturation	% saturation.

Enthalpy	kJ/kg

Moisture Content	g/m3 (grammes of moisture per cubic metre)

Moisture Content	kg/kg    * NEW ADDITION *

Dew Point	Celsius  * NEW ADDITION *

Barometric Pressure	kPa

Velocity Pressure	Pa

The velocity pressure calculator is useful when sizing velocity probes for ducts. The standard barometric pressure of 101.325 kPa can be changed via the keyboard, and later restored to it's original value by pressing the Default Reset button.

Copy calculations to Windows' clipboard

By selecting either Psych or Veloc, from the Copy menu, the Psychrometric or Pressure calculations respectively can be copied to the windows clipboard for use in other applications. Most "good" windows applications can paste the contents of the clipboard by pressing <Ctrl>-v together.

Printing calculations

Select the File menu, and select the Print sub menu. HVACalc will send the current set of results to the default printer. The output is restricted to text only, as this provides maximum compatibility with the widest range of printers.

Source of calculations

Calculations have been derived from those available in the British CIBSE (The Chartered Institute of Building Services Engineers) Guide, Volume C, Reference Data, section C1.

Registration:

This software is FREEWARE.

If you would like any more information then you can contact me on:

eclipse@dial.pipex.com

You are allowed to copy the original HVAC.EXE file as often as you wish. Please distribute this file freely to your friends, colleagues, Internet, local BBS's etc.

Personalised Distribution Licences

Impress your customers! If you would like a personalised copy of this software and a licence allowing you to distribute it freely to your clients as a promotional item, then please contact me for prices. Company logo's and small alterations to the program may also be possible.

Liability:

No liability can be accepted for any claim arising through the use of HVACalc howsoever caused. No guarantees are given or implied as to the accuracy of the calculations carried out by HVACalc.