Psion Series 3a NMEA reader program notes

b0.01 (preliminary) 29th January 1996

The program allows the Psion Organiser 3a to read NMEA data from the Rockwell (and other) GPS receivers.

Key options are :

| Psion-X | Exit program |  |
| --- | --- | --- |
| Psion-O | Odometer mode | (Not yet implemented) |
| Psion-A | Add place name to database | (Not yet implemented) |
| Psion-D | Configure DGPS port |  |
| Psion-N | Calculate UK NGR |  |
| Psion-W | Move window positions |  |

The program will ‘beep’ when a position is initially displayed, and when the status of accuracy of the displayed position changes, i.e. when differential corrections are applied, or when navigation is lost.

(screen shot from older version of program, new version has Psion status window)

The program is not yet complete, it is intended to add various functionality such as a place names database and graphical tracking.  The program at the moment sends strings to the NMEA GPS device to set up the messages, these will be made configurable in a later version.

Serial cable configuration

The program requires a one-to-one connection from the Psion 3 link serial adaptor to the TDC Microtracker serial board (as supplied in the GPS starter pack).  The one-to-one connection essentially re-instates the null modem device built into the standard serial cable supplied with the Psion 3 link.

input.doc  28 July, 2026